Robotic Manipulation – Exercise 5 – Report

The approach chosen for planning the grasp

First of all, I needed to calculate the angle of the friction cone. For that, I used the inverse of the tangent function over the friction coefficient (μ). Since it is assumed that the friction coefficient (μ) is 1 for all objects,

α=arctan⁡(μ)

After that I started coding the script, which has 7 differentiated steps:

- Constructing friction cone at each grasp point.

- Check all other grasping points that lie within the friction cone.

- Get all stable pairs of grasp points.

- Select optimal pair of grasp points.

- Compute approach angle for those points.

- Plot all.

- Write solution in a file.

So, I first calculate the friction cone for every one of the possible contact points. Then I check which other grasping points lie within that cone, being those potential stable pairs. After that, since in our case we are dealing with two contacts and a planar object, I check if both points of contact points are within the friction cones to see if the grasp made by that pair is stable. Then out of all these resultant stable pairs of contact points I choose the optimal one by following the metric explained in the later “bonus task” question. Finally I plot and save the solution.

How to run your grasp planning code

The script that compute the grasp planning is written in Python. It is required to execute the code in Python3.5 or later.

To execute the code just execute:  python3 src/grasp_planning.py . The script must be called from the root folder of the project (exercise5), otherwise an error will prompt. It could take up to a few minutes to finish, so please be patient. The code probably can be optimized in terms of time cost but I did not have enough time to work on that and, also, I believe it’s not the most important part here. I have a few prints in the code showing the progress of the script.

Visualization of the grasping points, approach direction, and friction cones on the object polygon

The first grasping point is represented with the green color, and second with red.

*The script might output different results due to the metric that I chose to use.

**NOTE: I noticed that the order of the points is important here, in case that you receive a planning error (if you do) just change the order of the points in the file grasp_data.txt. I did not put this consideration in the code because I considered that what is asked in this exercise is to give two stable grasping points, and not also consider if those points are reachable by the robot arms.

Is the grasp both force-closure and form-closure, either of, or neither? Explain why.

Force-closure means that a grasp is able to cancel any external wrench if contact forces are sufficiently high. In our case the grasp is in force closure because for any external wrench applied there exists contact wrenches that cancel it.

And then for it to be form-closure we would need a grasp whose contacts are placed in a way that makes any motion of the object impossible. With only two contact points you can always find a motion that can be made. Which means that if for example the motors cannot give enough forces to counter external forces such as gravity the polygon would fall.

All form-closure grasps are also force-closure and not the opposite way.

If the object was not force-closure, where would we need to place additional contacts to achieve it?

As mentioned in the answer of the previous question, force-closure means that a grasp is able to cancel any external wrench if contact forces are sufficiently high.

We would need to place the additional contacts in such a way that the contact forces should be pointing into the friction cones of the other points of contact. Furthermore, we should place these contacts in the position where grasp quality metrics are maximized.

For the bonus task specifically report the approach and the metric used for planning the stable grasp

I chose to use the minimum distance from the grasping points to the center of gravity of the polygon (centroid) as the metric for optimality.

The entire approach, already described as an answer of the first question, is the following. I first calculate the friction cone for every one of the possible grasping points. Then I check which other grasping points lie within that cone, being those potential stable pairs. After that, since in our case we are dealing with two contacts and a planar object, I check if both points of contact points are within the friction cones to see if the grasp made by that pair is stable. Then out of all these resultant stable pairs of contact points I choose the optimal one by following the metric given in the previous paragraph.

Estimate of time spent on this exercise

16 hours